Create Smartgroup and Bind Member Port by Dynamic Mode

Create smartgroup

ZXAN(config)#interface smartgroup1

ZXAN(config-if-smartgroup1)#exit

Configure smartgroup dynamic mode

ZXAN(config)#lacp

ZXAN(config-lacp)#interface smartgroup1

ZXAN(config-lacp-sg-if-smartgroup1)#lacp mode 802.3ad  -------Configure smartgroup dynamic mode

ZXAN(config-lacp-sg-if-smartgroup1)#lacp load-balance src-mac --Configure the load balance mode

ZXAN(config-lacp-sg-if-smartgroup1)#exit

Member port binds to smartgroup

ZXAN(config-lacp)#interface xgei-1/18/1

ZXAN(config-lacp-member-if-xgei-1/18/1)#smartgroup 1 mode active ---For the dynamic mode, at least one of the local and peer member ports must join the smartgroup as active

ZXAN(config-lacp-member-if-xgei-1/18/1)#exit

ZXAN(config-lacp)#interface xgei-1/19/1

ZXAN(config-lacp-member-if-xgei-1/19/1)#smartgroup 1 mode active ----For the dynamic mode, at least one of the local and peer member ports must join the smartgroup as active

ZXAN(config-lacp-member-if-xgei-1/19/1)#exit

Enable the management status of the uplink port

ZXAN(config-lacp)#exit

ZXAN(config)#interface xgei-1/18/1

ZXAN(config-if-xgei-1/18/1)#no shutdown --------------------------The uplink port and PON port are shutdown by default, first, you need to configure no shutdown to be able to use

ZXAN(config-if-xgei-1/18/1)#exit

ZXAN(config)#interface xgei-1/19/1

ZXAN(config-if-xgei-1/19/1)#no shutdown --------------------------The uplink port and PON port are shutdown by default, first, you need to configure no shutdown to be able to use.

ZXAN(config-if-xgei-1/19/1)#exit

If LACP is configured for both parties and the interconnected successfully, the LACP status is shown as below:

ZXAN#show lacp 1 counters

Smartgroup:1

Actor         LACPDUs               Marker    LACPDUs    Marker

Port          Tx         Rx         Tx  Rx    Err        Err

-------------------------------------------------------------------

xgei-1/18/1   2605       2548       0   0     0          0

xgei-1/19/1   2605       2556       0   0     0          0

ZXAN#show lacp 1 internal

Smartgroup:1

Flags:              * - Port is Active member Port

S - Port is requested in Slow LACPDUs

F - Port is requested in Fast LACPDUs

A - Port is in Active mode

P - Port is in Passive mode

Actor               Agg      LACPDUs  Port  Oper   Port  RX            Mux

Port[Flags]         State    Interval Pri   Key    State Machine       Machine

--------------------------------------------------------------------------------

xgei-1/18/1[SA*]    ACTIVE   30       32768 0x121  0x3d  CURRENT       COLL&DIST

xgei-1/19/1[SA*]    ACTIVE   30       32768 0x121  0x3d  CURRENT       COLL&DIST

ZXAN#show lacp 1 neighbors

Smartgroup 1  neighbors

Actor         Actor     Partner               Partner   Port      Oper    Port

Port          Port No.  System ID             Port No.  Priority  Key     State

-------------------------------------------------------------------------------

xgei-1/18/1   4609      0x8000,cc1a.faea.0ee0 8276      32768     0x1f21  0x3d

xgei-1/19/1   4865      0x8000,cc1a.faea.0ee0 8290      32768     0x1f21  0x3d